RENEWABLE ENERGY INTELLIGENCE

Predictive Maintenance for Solar & Wind Assets

Project Explanation & Working Model Documentation

# 1. Project Overview

Renewable Energy Intelligence is a predictive-maintenance platform for solar panels and wind turbines. It collects sensor readings, analyzes the readings using Python-based machine learning, identifies abnormal behavior, assigns a risk level to each asset, prioritizes maintenance work, and estimates the energy and revenue that could be lost if maintenance is delayed.

The main idea is simple: instead of waiting for an asset to fail, the system continuously watches its condition and gives the operator an early warning.

# 2. Problem Statement

Unplanned downtime in solar and wind farms can reduce energy generation and revenue. Manual inspection of a large number of panels or turbines is slow, expensive, and may miss early signs of degradation. The proposed system uses simulated or real IoT sensor data to detect anomalies and degradation before a serious failure occurs.

Detect abnormal sensor behavior early.

Flag assets that need attention.

Rank maintenance jobs by urgency.

Estimate the energy/revenue impact of doing nothing.

Give technicians a clear dashboard instead of raw sensor data.

# 3. Project Objectives

Monitor solar and wind assets continuously.

Create a reliable sensor-data pipeline.

Detect anomalies using machine learning.

Convert model output into understandable risk levels.

Prioritize maintenance visits.

Estimate potential energy and revenue loss.

Provide dashboards for operators and technicians.

Store historical readings, alerts, predictions, and maintenance records.

# 4. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Frontend | React + TypeScript | Operator/technician dashboards, charts, asset status, alerts |
| Backend | Flask + Python | REST APIs, business logic, authentication, ML integration |
| Database | PostgreSQL + SQL | Assets, sensor readings, predictions, alerts, maintenance history |
| AI/ML | Pandas + NumPy + scikit-learn | Data preparation, anomaly detection, feature engineering, risk scoring |
| IoT | Sensor simulation / integration | Generate or receive vibration, temperature, current, soiling and other readings |

# 5. High-Level Architecture

The system can be explained as five connected layers:

Sensor/IoT Layer → generates or receives asset readings.

Data Layer → validates and stores readings in PostgreSQL.

ML Layer → cleans data, creates features, detects anomalies and calculates risk.

Backend/API Layer → exposes asset health, alerts, priorities and loss estimates through Flask APIs.

Frontend Layer → displays dashboards for operators and technicians using React + TypeScript.

Simple architecture flow:

Sensors → Data Validation → PostgreSQL → Pandas/NumPy → ML Anomaly Detection → Risk & Loss Engine → Flask REST API → React Dashboard → Technician Action → Maintenance Feedback

# 6. Sensor Data Used

| Asset | Example Sensors | What it can indicate |
| --- | --- | --- |
| Solar panel/string | Panel temperature, current, voltage, irradiance, soiling level | Overheating, reduced output, soiling, electrical degradation |
| Wind turbine | Vibration, gearbox temperature, generator temperature, wind speed, power output | Bearing/gearbox issues, overheating, abnormal mechanical behavior |
| Both | Power output, timestamp, asset ID, location | Performance comparison and historical trend analysis |

# 7. Proper Working Flow

Step 1 — Asset Registration: The operator registers assets such as Solar Panel SP-101 or Wind Turbine WT-07. Each asset has an ID, type, location, rated power and operating information.

Step 2 — Sensor Data Collection: Sensor readings arrive at regular intervals. For a prototype, Python can simulate realistic readings and occasional abnormal patterns.

Step 3 — Data Validation: Flask/Python checks missing values, impossible values, timestamps and sensor ranges. Invalid records are flagged or cleaned before ML processing.

Step 4 — Store Data: Clean readings are stored in PostgreSQL so the system can build historical trends.

Step 5 — Feature Engineering: Pandas/NumPy calculate useful features such as rolling average temperature, temperature change, vibration average, vibration standard deviation, current deviation, power deviation and soiling trend.

Step 6 — ML Anomaly Detection: The trained scikit-learn model learns normal operating behavior. A new reading is compared with that normal pattern. The model produces an anomaly score or anomaly flag.

Step 7 — Health/Risk Scoring: The anomaly result is combined with sensor severity, persistence, asset criticality and performance drop to calculate a health score/risk level.

Step 8 — Energy Loss Estimation: The system estimates how much generation could be lost if the issue continues, then converts the expected energy loss into an approximate revenue loss.

Step 9 — Alert & Priority: The backend creates an alert and assigns priority such as Low, Medium, High or Critical. High-risk assets move to the top of the technician work queue.

Step 10 — Dashboard: React displays asset health, sensor trends, anomaly reason, risk level, estimated loss and recommended action.

Step 11 — Maintenance Feedback: After inspection/repair, the technician records the action. This becomes historical information for reporting and future model improvement.

# 8. Example — Wind Turbine Prediction

Assume Wind Turbine WT-07 normally operates with a gearbox temperature around 55–65°C and low, stable vibration. During one period, the system receives:

| Sensor | Normal/Expected | Observed |
| --- | --- | --- |
| Gearbox temperature | 55–65°C | 78°C |
| Vibration RMS | Low/stable | Much higher than baseline |
| Power output | Near expected for wind speed | 10–15% below expected |
| Wind speed | Stable | Normal |

The important point is that high temperature alone does not automatically mean failure. The model looks at the combination of features and whether the abnormal pattern persists. Because temperature is high, vibration is elevated, and power is lower than expected for the available wind, the combined pattern is unusual.

Example model result: anomaly = 1, anomaly score = high, health score = 28/100. The business layer converts this into Risk = CRITICAL and creates a maintenance alert.

The dashboard could show: WT-07 → Critical → suspected gearbox/mechanical degradation → power impact 12% → estimated loss if ignored: ₹X/day → recommended action: inspect gearbox/bearings as soon as possible.

The model is not claiming that a gearbox has definitely failed. It is saying that the sensor pattern is abnormal enough to justify inspection. A technician confirms the actual fault.

# 9. Example — Solar Panel Soiling

Suppose Solar Panel/String SP-101 has normal irradiance but its power output gradually decreases while the soiling sensor increases. Temperature and current may also move away from their expected relationship.

The system detects a persistent performance deviation. Instead of labeling the panel as mechanically failed, the dashboard can classify it as a lower-severity performance issue such as probable soiling. The recommended action could be cleaning/inspection.

This demonstrates why the system should use asset-specific rules and model features: different sensor patterns can represent different maintenance situations.

# 10. Anomaly Detection Model

An unsupervised model such as Isolation Forest is a strong starting point because failure labels are usually limited. The model is trained mainly on examples of normal operation.

Example input features:

temperature

temperature_rate_of_change

vibration_rms

vibration_std

current

current_deviation_from_baseline

power_output

power_deviation_from_expected

soiling_level (solar)

wind_speed (wind)

asset_age or operating_hours

Conceptual pipeline: historical normal data → feature engineering → Isolation Forest training → new sensor window → anomaly score → risk engine.

A supervised model can be added later if the project has enough labeled maintenance/failure records. The important design principle is to separate anomaly detection from the final maintenance decision.

# 11. Risk Score & Priority

A simple prototype can combine multiple signals into a transparent score. For example:

Risk Score = 40% × anomaly severity + 25% × performance loss + 20% × persistence + 15% × asset criticality

This is an example design, not a fixed scientific formula. The weights can be tuned using validation data and domain knowledge.

0–30 → Low: continue monitoring

31–60 → Medium: inspect during planned maintenance

61–80 → High: schedule maintenance soon

81–100 → Critical: prioritize immediate inspection

Keeping this layer separate from the ML model makes the system easier to explain to evaluators and technicians.

# 12. Energy & Revenue Loss Estimation

The platform should answer: 'What could happen if we do not act?' A simple estimate can be based on expected power, predicted performance reduction and duration.

Estimated energy loss (kWh) ≈ expected power (kW) × predicted loss fraction × expected affected hours

Estimated revenue loss ≈ estimated energy loss (kWh) × electricity tariff (₹/kWh)

Example: if a turbine is expected to produce 500 kW during an affected 6-hour period, the predicted loss is 12%, and the tariff is ₹5/kWh:

Energy loss ≈ 500 × 0.12 × 6 = 360 kWh. Revenue loss ≈ 360 × ₹5 = ₹1,800 for that period.

In a real deployment, the calculation should account for wind/irradiance forecasts, asset availability, curtailment, tariff structure and uncertainty.

# 13. Project Explanation

“Our project is a Renewable Energy Intelligence platform for predictive maintenance of solar panels and wind turbines. Sensors continuously provide values such as temperature, vibration, current, power and soiling. The backend validates and stores this data in PostgreSQL. Python with Pandas and NumPy creates useful features, and a scikit-learn anomaly detection model learns normal asset behavior. When a new pattern becomes abnormal, the system calculates a health and risk score. It then prioritizes the asset for maintenance and estimates the energy and revenue that may be lost if the issue is ignored. React and TypeScript provide dashboards for operators and technicians, while Flask exposes the APIs. The key benefit is moving from reactive maintenance after failure to predictive maintenance before failure.””

# 14. Final End-to-End Flow

USER / OPERATOR → REACT DASHBOARD → FLASK API → POSTGRESQL / SENSOR DATA → PANDAS + NUMPY FEATURE ENGINEERING → SCIKIT-LEARN ANOMALY DETECTION → HEALTH & RISK SCORE → ENERGY/REVENUE LOSS ESTIMATION → ALERT PRIORITY → TECHNICIAN MAINTENANCE → FEEDBACK/HISTORY

Core message: SENSE → STORE → ANALYZE → PREDICT → PRIORITIZE → ACT → LEARN